Three design factors in this experiment:

| code | paper height(mm) | width-height ratio | leg length(mm) |
| --- | --- | --- | --- |
| -1 | 148.5 | 0.4 | 8 |
| 1 | 210 | 0.5 | 10 |

Unwanted noises:

- The material property of papers may vary in each frog.

- The humidity in air will affect the elasticity of paper.

- The angle between the finger and a frog will vary in each jumping test.

- The friction coefficient between frog and finger may change because of sweat on hand.

- The finger’s force and placement on a frog will vary in each jumping test.

- X1: paper height

- X2: width-height ratio

- X3: leg length

23 factorial experiment:

| test | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 1 | -1 | -1 | -1 |
| 2 | 1 | -1 | -1 |
| 3 | -1 | 1 | -1 |
| 4 | 1 | 1 | -1 |
| 5 | -1 | -1 | 1 |
| 6 | 1 | -1 | 1 |
| 7 | -1 | 1 | 1 |
| 8 | 1 | 1 | 1 |

In my experiment, I will create three identical frogs for each combination of factors. To ensure randomization, I will randomize the order of the jumping trials instead of following a fixed sequence for each run. Since there are three runs, each frog will perform three jumps.

Experiment table:

How to handle the noises:

- To prevent I using my fingernails to press paper frogs, I always cut my nails before I do experiment.

- Do more replication and randomize the experimental trials to make noises more uniformly and more randomly distributed.

Experiment details:

- Day: 6/10 Monday

- time: 11:00pm~12:30pm

- place: home

- witness: 王邑宇 (reason: he is my brother)

measurement method:

- Measurement plate: Design a plate with markings ranging from 0 to 55 cm (as the picture in 4.)

- Align frogs: Position each paper frog so that its rear edge aligns with the zero line on the measurement plate before each jump.

- Execute a jump: Use fingertip to press down on the back of the frog and release it to initiate the jump

- Valid jumps: If the frog rolls or bounces twice or more, disregard the result and perform the jump again.

- Distance measurement: For each valid jump, measure the shortest distance between the zero line and the landing point of the frog. This distance is the result for that trial.

experiment result:

Parameters of this experiment: m=8, n=9⇒N=72

Average and sample variance in each test: yi=j=1nyjn,  si2=j=1nyj-y2n-1

Effect coefficient:

| E1 | E2 | E3 | E12 | E23 | E13 | E123 |
| --- | --- | --- | --- | --- | --- | --- |
| -69.4444 | -38.8889 | 9.444444 | 52.22222 | 25.55556 | -15 | 6.111111111 |

Assuming all effects are null effect: H0:Ei=0 ,  H1:Ei≠0 ,  α=0.05

Degree of freedom for t-test: ν=mn-1=8×9-1=64

tEi=Ei-0seffect<t64,0.025 or tEi=Ei-0seffect>t64,0.975⇒reject H0

where t64,0.025=-1.997729654, t64,0.975=1.997729654

seffect2=VarE=4sp2mn=47.63695988,  sp2=imsi2m=857.4652778

|  |  | t-test | p-value | t_64,0.025 | t_64,0.975 |
| --- | --- | --- | --- | --- | --- |
| E1 | -69.4444 | -10.0616 | 8.28E-15 | -1.99773 | 1.997729654 |
| E2 | -38.8889 | -5.63448 | 4.23E-07 | -1.99773 | 1.997729654 |
| E3 | 9.444444 | 1.368373 | 0.17598 | -1.99773 | 1.997729654 |
| E12 | 52.22222 | 7.566296 | 1.88E-10 | -1.99773 | 1.997729654 |
| E23 | 25.55556 | 3.702656 | 0.000446 | -1.99773 | 1.997729654 |
| E13 | -15 | -2.1733 | 0.033465 | -1.99773 | 1.997729654 |
| E123 | 6.111111 | 0.885418 | 0.379246 | -1.99773 | 1.997729654 |

E1,E2,E12,E23,E13 reject H0, they are significant factors.

y=b0+b1x1+b2x2+b12x1x2+b23x2x3+b13x1x3

=214.4444444-34.72222222x1-19.44444444x2+26.11111111x1x2+12.77777778x2x3-7.5x1x3

| source of variation | SS | DOF | MS | F | F0.05 |
| --- | --- | --- | --- | --- | --- |
| E1 | 86805.56 | 1 | 86805.56 | 100.2381 | 3.986269 |
| E2 | 27222.22 | 1 | 27222.22 | 31.43468 | 3.986269 |
| E12 | 49088.89 | 1 | 49088.89 | 56.68507 | 3.986269 |
| E23 | 11755.56 | 1 | 11755.56 | 13.57465 | 3.986269 |
| E13 | 4050 | 1 | 4050 | 4.676711 | 3.986269 |
| error | 57155.56 | 66 | 865.9933 |  |  |
| total | 236077.8 | 71 |  |  |  |

R2=SSRSST=SSE1+SSE2+SSE12+SSE23+SSE13SST=0.757895232

adjusted R2=1-SSE/dofSST/dof=57155.55556/66236077.7778/71=0.739553962

Suppose that residuals are normal distributed. The parameters of normal distribution are e and σe:

e=i=18j=19eij72=-5.92119E-15

σe=i8j9eij-e272-1=35.4757516

- Rank each residual from 1~72

- The c.d.f. of i th effect is: pi=(i-0.5)/72

- The normal distribution value of i th effect: N-1(pi;e,σe)

Plot the residuals vs. run orders:

Plot the residuals vs. y:

Plot the residuals vs. y:

Plot the residuals vs. x1:

Plot the residuals vs. x2:

Plot the residuals vs. x3:

Bartlett’s test (α=0.05):

H0:σ12=σ22=⋯=σm2=σϵ2

H1:at least one σa2≠σb2 , a≠b

Reject H0 if Xcalc2>Xm-1,α2=X7,0.052:

Xcalc2=Mc

M=N-mln sp2-i=1mni-1ln si2 , sp2=imni-1si2N-m

c=1+13(m-1)[i=1m1ni-1-1N-m]

⇒Xcalc2=7.682931678 , X7,0.052=14.06714045

⇒ Xcalc2<Xm-1,α2 , accept H0

SN ratio: Larger-the-best

η=-10log101ni=1n1Xi2

η=b0+b2x2+b12x1x2=42.93968312-0.90043377x2+1.548099668x1x2