Three design factors in this experiment:

| code | paper height(mm) | width-height ratio | leg length(mm) |
| --- | --- | --- | --- |
| -1 | 148.5 | 0.4 | 8 |
| 1 | 210 | 0.5 | 10 |

Unwanted noises:

- The material property of papers may vary in each frog.

- The humidity in air will affect the elasticity of paper.

- The angle between the finger and a frog will vary in each jumping test.

- The friction coefficient between frog and finger may change because of sweat on hand.

- The finger’s force and placement on a frog will vary in each jumping test.

- X1: paper height

- X2: width-height ratio

- X3: leg length

23 factorial experiment:

| test | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 1 | -1 | -1 | -1 |
| 2 | 1 | -1 | -1 |
| 3 | -1 | 1 | -1 |
| 4 | 1 | 1 | -1 |
| 5 | -1 | -1 | 1 |
| 6 | 1 | -1 | 1 |
| 7 | -1 | 1 | 1 |
| 8 | 1 | 1 | 1 |

In my experiment, I will create three identical frogs for each combination of factors. To ensure randomization, I will randomize the order of the jumping trials instead of following a fixed sequence for each run. Since there are three runs, each frog will perform three jumps.

Experiment table:

How to handle the noises:

- To prevent I using my fingernails to press paper frogs, I always cut my nails before I do experiment.

- Do more replication and randomize the experimental trials to make noises more uniformly and more randomly distributed.

Experiment details:

- Day: 6/10 Monday

- time: 11:00am~12:30pm

- place: home

- witness: 王邑宇 (reason: he is my brother)

measurement method:

- Measurement plate: Design a plate with markings ranging from 0 to 55 cm (as the picture in 4.)

- Align frogs: Position each paper frog so that its rear edge aligns with the zero line on the measurement plate before each jump.

- Execute a jump: Use fingertip to press down on the back of the frog and release it to initiate the jump

- Valid jumps: If the frog rolls or bounces twice or more, disregard the result and perform the jump again.

- Distance measurement: For each valid jump, measure the shortest distance between the zero line and the landing point of the frog. This distance is the result for that trial.

experiment result:

Parameters of this experiment: m=8, n=9⇒N=72

Average and sample variance in each test: yi=j=1nyjn,  si2=j=1nyj-y2n-1

|  |  |  |  | replicate 1 |  |  | replicate 2 |  |  | replicate 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | x1 | x2 | x3 | run1 | run2 | run3 | run1 | run2 | run3 | run1 | run2 | run3 | y_bar | s_i^2 |
| 1 | -1 | -1 | -1 | 255 | 300 | 285 | 240 | 310 | 300 | 280 | 300 | 360 | 292.2222 | 1175.694 |
| 2 | 1 | -1 | -1 | 200 | 205 | 230 | 220 | 155 | 200 | 150 | 165 | 200 | 191.6667 | 806.25 |
| 3 | -1 | 1 | -1 | 150 | 175 | 160 | 190 | 150 | 190 | 220 | 195 | 205 | 181.6667 | 606.25 |
| 4 | 1 | 1 | -1 | 170 | 210 | 160 | 160 | 170 | 185 | 150 | 155 | 200 | 173.3333 | 431.25 |
| 5 | -1 | -1 | 1 | 300 | 290 | 345 | 230 | 285 | 270 | 275 | 295 | 385 | 297.2222 | 1994.444 |
| 6 | 1 | -1 | 1 | 140 | 150 | 145 | 200 | 150 | 185 | 145 | 140 | 135 | 154.4444 | 502.7778 |
| 7 | -1 | 1 | 1 | 175 | 210 | 230 | 230 | 210 | 245 | 245 | 210 | 275 | 225.5556 | 815.2778 |
| 8 | 1 | 1 | 1 | 180 | 175 | 205 | 200 | 250 | 205 | 200 | 175 | 205 | 199.4444 | 527.7778 |

Effect coefficient:

| E1 | E2 | E3 | E12 | E23 | E13 | E123 |
| --- | --- | --- | --- | --- | --- | --- |
| -69.4444 | -38.8889 | 9.444444 | 52.22222 | 25.55556 | -15 | 6.111111111 |

Assuming all effects are null effect: H0:Ei=0 ,  H1:Ei≠0 ,  α=0.05

Degree of freedom for t-test: ν=mn-1=8×9-1=64

tEi=Ei-0seffect,  tEi<t64,0.025 or  tEi>t64,0.975⇒reject H0

where t64,0.025=-1.997729654, t64,0.975=1.997729654

seffect2=VarE=4sp2mn=47.63695988,  sp2=imsi2m=857.4652778

|  |  | t-test | p-value | t_64,0.025 | t_64,0.975 |
| --- | --- | --- | --- | --- | --- |
| E1 | -69.4444 | -10.0616 | 8.28E-15 | -1.99773 | 1.997729654 |
| E2 | -38.8889 | -5.63448 | 4.23E-07 | -1.99773 | 1.997729654 |
| E3 | 9.444444 | 1.368373 | 0.17598 | -1.99773 | 1.997729654 |
| E12 | 52.22222 | 7.566296 | 1.88E-10 | -1.99773 | 1.997729654 |
| E23 | 25.55556 | 3.702656 | 0.000446 | -1.99773 | 1.997729654 |
| E13 | -15 | -2.1733 | 0.033465 | -1.99773 | 1.997729654 |
| E123 | 6.111111 | 0.885418 | 0.379246 | -1.99773 | 1.997729654 |

E1,E2,E12,E23,E13 reject H0, they are significant factors.

y=b0+b1x1+b2x2+b12x1x2+b23x2x3+b13x1x3

=214.4444444-34.72222222x1-19.44444444x2+26.11111111x1x2+12.77777778x2x3-7.5x1x3

| source of variation | SS | DOF | MS | F | F0.05 |
| --- | --- | --- | --- | --- | --- |
| E1 | 86805.56 | 1 | 86805.56 | 100.2381 | 3.986269 |
| E2 | 27222.22 | 1 | 27222.22 | 31.43468 | 3.986269 |
| E12 | 49088.89 | 1 | 49088.89 | 56.68507 | 3.986269 |
| E23 | 11755.56 | 1 | 11755.56 | 13.57465 | 3.986269 |
| E13 | 4050 | 1 | 4050 | 4.676711 | 3.986269 |
| error | 57155.56 | 66 | 865.9933 |  |  |
| total | 236077.8 | 71 |  |  |  |

R2=SSRSST=SSE1+SSE2+SSE12+SSE23+SSE13SST=0.757895232

adjusted R2=1-SSE/dofSST/dof=57155.55556/66236077.7778/71=0.739553962

Suppose that residuals are normal distributed. The parameters of normal distribution are e and σe:

e=i=18j=19eij72=-5.92119E-15

σe=i8j9eij-e272-1=35.4757516

- Rank each residual from 1~72

- The c.d.f. of i th effect is: pi=(i-0.5)/72

- The normal distribution value of i th effect: N-1(pi;e,σe)

Residual Q-Q plot:

Plot the residuals vs. run orders:

Plot the residuals vs. y:

Plot the residuals vs. y:

Plot the residuals vs. x1:

Plot the residuals vs. x2:

Plot the residuals vs. x3:

Bartlett’s test (α=0.05):

H0:σ12=σ22=⋯=σm2=σϵ2

H1:at least one σa2≠σb2 , a≠b

Reject H0 if Xcalc2>Xm-1,α2=X7,0.052:

Xcalc2=Mc

M=N-mln sp2-i=1mni-1ln si2 , sp2=imni-1si2N-m

c=1+13(m-1)[i=1m1ni-1-1N-m]

⇒Xcalc2=7.682931678 , X7,0.052=14.06714045

⇒ Xcalc2<Xm-1,α2 , accept H0

SN ratio: Larger-the-best

η=-10log101ni=1n1Xi2

|  |  |  |  | replicate 1 |  |  | replicate 2 |  |  | replicate 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | x1 | x2 | x3 | run1 | run2 | run3 | run1 | run2 | run3 | run1 | run2 | run3 | SN |
| 1 | -1 | -1 | -1 | 255 | 300 | 285 | 240 | 310 | 300 | 280 | 300 | 360 | 49.15775 |
| 2 | 1 | -1 | -1 | 200 | 205 | 230 | 220 | 155 | 200 | 150 | 165 | 200 | 45.37338 |
| 3 | -1 | 1 | -1 | 150 | 175 | 160 | 190 | 150 | 190 | 220 | 195 | 205 | 44.96722 |
| 4 | 1 | 1 | -1 | 170 | 210 | 160 | 160 | 170 | 185 | 150 | 155 | 200 | 44.62473 |
| 5 | -1 | -1 | 1 | 300 | 290 | 345 | 230 | 285 | 270 | 275 | 295 | 385 | 49.21638 |
| 6 | 1 | -1 | 1 | 140 | 150 | 145 | 200 | 150 | 185 | 145 | 140 | 135 | 43.57489 |
| 7 | -1 | 1 | 1 | 175 | 210 | 230 | 230 | 210 | 245 | 245 | 210 | 275 | 46.87 |
| 8 | 1 | 1 | 1 | 180 | 175 | 205 | 200 | 250 | 205 | 200 | 175 | 205 | 45.85851 |

Effect coefficient:

| E1 | E2 | E3 | E12 | E23 | E13 | E123 |
| --- | --- | --- | --- | --- | --- | --- |
| -2.694963152 | -1.250482833 | 0.349173936 | 2.017970648 | 1.219103481 | -0.631532694 | 0.297030052 |

Assuming all effects are null effects, and estimated standard error by the lowest three effects. H0:Ei=0 ,  H1:Ei≠0 ,  α=0.05

E13 and E123 are relatively small, therefore, they are used for testing the statistical significance of main effects and larger interaction effects.

seffect2=E13-02+E123-022

tEi=Ei-0seffect,  tEi<t2,0.025 or  tEi>t2,0.975⇒reject H0

where t2,0.025=-4.30265273, t2,0.975=4.30265273

|  | effect | t-test | p-value | t_2,0.025 | t_2,0.975 |
| --- | --- | --- | --- | --- | --- |
| E1 | -2.694963152 | -5.461053371 | 0.031934 | -4.30265 | 4.302653 |
| E2 | -1.250482833 | -2.533969152 | 0.126788 | -4.30265 | 4.302653 |
| E3 | 0.349173936 | 0.707563479 | 0.552555 | -4.30265 | 4.302653 |
| E12 | 2.017970648 | 4.089200775 | 0.054923 | -4.30265 | 4.302653 |
| E23 | 1.219103481 | 2.470382266 | 0.132146 | -4.30265 | 4.302653 |
| E13 | -0.631532694 | -1.279733174 | 0.329027 | -4.30265 | 4.302653 |
| E123 | 0.297030052 | 0.601899497 | 0.608386 | -4.30265 | 4.302653 |

It shows that only E1 is significant effect to the SN ratio. Consequently, the predictive model of SN ratio is:

η=b0+b1x1=46.20535766-1.347481576x1

Based on the predictive model of jumping distance of paper frogs, a best combination of three factor can be find:

y =214.4444444-34.72222222x1-19.44444444x2+26.11111111x1x2+12.77777778x2x3-7.5x1x3

| x1 | x2 | x3 | y | x1 | x2 | x3 | y |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -1 | -1 | -1 | 300 | -1 | -1 | 1 | 289.4444 |
| 1 | -1 | -1 | 193.3333 | 1 | -1 | 1 | 152.7778 |
| -1 | 1 | -1 | 183.3333 | -1 | 1 | 1 | 223.8889 |
| 1 | 1 | -1 | 181.1111 | 1 | 1 | 1 | 191.6667 |

The optimum setting of three factors is x1,x2,x3=-1,-1,-1

On the other hand, the predictive model of SN ratio is:

η=46.20535766-1.347481576x1

| x1 | η |
| --- | --- |
| -1 | 47.55283923 |
| 1 | 44.85787608 |

It shows that when x1=-1, the jumping distance SN ratio of paper frogs is largest.

As a result, there is no conflict between the predictive model of jumping distance and the predictive model of the SN ratio for jumping distance. The estimated η​ indicates that the SN ratio performs best when x1=-1. Similarly, the estimated y shows that the optimal jumping distance occurs when x1,x2,x3=-1,-1,-1.

y​x1=-1,x2=-1,x3=-1=214.4444444-34.72222222x1-19.44444444x2+26.11111111x1x2+12.77777778x2x3-7.5x1x3

=300

Make three optimum frogs and measure their jumping distance:

|  | replication1 | replication2 | replication3 |
| --- | --- | --- | --- |
| run1 distance(mm) | 270 | 330 | 360 |
| run2 distance(mm) | 290 | 275 | 340 |
| run3 distance(mm) | 355 | 310 | 330 |

The average jumping distance of the frogs at the optimal conditions was measured to be 317.78 mm. This observed value is closely aligned with the predictions made by the model.

| test | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 1 | -1 | -1 | -1 |
| 2 | 1 | -1 | -1 |
| 3 | -1 | 1 | -1 |
| 4 | 1 | 1 | -1 |
| 5 | -1 | -1 | 1 |
| 6 | 1 | -1 | 1 |
| 7 | -1 | 1 | 1 |
| 8 | 1 | 1 | 1 |
| 9 | -1.682 | 0 | 0 |
| 10 | 1.682 | 0 | 0 |
| 11 | 0 | -1.682 | 0 |
| 12 | 0 | 1.682 | 0 |
| 13 | 0 | 0 | -1.682 |
| 14 | 0 | 0 | 1.682 |
| 15 | 0 | 0 | 0 |

Experiment details:

- Day: 6/10 Monday

- time: 1:30pm~2:30pm

- place: home

- witness: 王邑宇 (reason: he is my brother)

measurement method: same method in (5)

experiment result:

Parameters of this experiment: m=15, n=9⇒N=135

Average and sample variance in each test: yi=j=1nyjn,  si2=j=1nyj-y2n-1

|  |  |  |  | Replication1 |  |  | Replication2 |  |  | Replication3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | x1 | x2 | x3 | run1 | run2 | run3 | run1 | run2 | run3 | run1 | run2 | run3 | y_bar | s_i^2 |
| 1 | -1 | -1 | -1 | 255 | 300 | 285 | 240 | 310 | 300 | 280 | 300 | 360 | 292.2222 | 1175.694 |
| 2 | 1 | -1 | -1 | 200 | 205 | 230 | 220 | 155 | 200 | 150 | 165 | 200 | 191.6667 | 806.25 |
| 3 | -1 | 1 | -1 | 150 | 175 | 160 | 190 | 150 | 190 | 220 | 195 | 205 | 181.6667 | 606.25 |
| 4 | 1 | 1 | -1 | 170 | 210 | 160 | 160 | 170 | 185 | 150 | 155 | 200 | 173.3333 | 431.25 |
| 5 | -1 | -1 | 1 | 300 | 290 | 345 | 230 | 285 | 270 | 275 | 295 | 385 | 297.2222 | 1994.444 |
| 6 | 1 | -1 | 1 | 140 | 150 | 145 | 200 | 150 | 185 | 145 | 140 | 135 | 154.4444 | 502.7778 |
| 7 | -1 | 1 | 1 | 175 | 210 | 230 | 230 | 210 | 245 | 245 | 210 | 275 | 225.5556 | 815.2778 |
| 8 | 1 | 1 | 1 | 180 | 175 | 205 | 200 | 250 | 205 | 200 | 175 | 205 | 199.4444 | 527.7778 |
| 9 | -1.682 | 0 | 0 | 210 | 220 | 215 | 145 | 160 | 150 | 165 | 175 | 175 | 179.4444 | 815.2778 |
| 10 | 1.682 | 0 | 0 | 195 | 205 | 215 | 170 | 190 | 165 | 155 | 165 | 170 | 181.1111 | 429.8611 |
| 11 | 0 | -1.682 | 0 | 205 | 185 | 180 | 175 | 185 | 225 | 225 | 180 | 205 | 196.1111 | 379.8611 |
| 12 | 0 | 1.682 | 0 | 200 | 190 | 205 | 225 | 195 | 220 | 240 | 235 | 220 | 214.4444 | 315.2778 |
| 13 | 0 | 0 | -1.682 | 205 | 190 | 220 | 210 | 230 | 200 | 175 | 155 | 170 | 195 | 606.25 |
| 14 | 0 | 0 | 1.682 | 195 | 220 | 210 | 230 | 245 | 245 | 225 | 230 | 240 | 226.6667 | 275 |
| 15 | 0 | 0 | 0 | 250 | 230 | 260 | 210 | 260 | 225 | 200 | 200 | 220 | 228.3333 | 562.5 |

Check the correlation coefficient between each factor (including quadratic terms):

|  | x1 | x2 | x3 | x1x2 | x2x3 | x1x3 | x1x2x3 | x1^2 | x2^2 | x3^2 | y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x1 | 1 |  |  |  |  |  |  |  |  |  |  |
| x2 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| x3 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |
| x1x2 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |
| x2x3 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |
| x1x3 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |
| x1x2x3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |
| x1^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |
| x2^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.3834 | 1 |  |  |
| x3^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.3834 | -0.3834 | 1 |  |
| y | -0.4146 | -0.18805 | 0.137271 | 0.411538 | 0.201391 | -0.11821 | 0.048159 | -0.19736 | 0.034362 | 0.085857 | 1 |

The result shows that there is little correlation between each factor, therefore, it can be considered as no multicollinearity effect among this regression models:

y=b0+b1x1+b2x2+b3x3+b12x1x2+b23x2x3+b13x1x3+b123x1x2x3+b12x12+b22x22+b32x32

Regression analysis results:

| 摘要輸出 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 迴歸統計 |  |  |  |  |  |  |
| R 的倍數 | 0.702658 |  |  |  |  |  |
| R 平方 | 0.493728 |  |  |  |  |  |
| 調整的 R 平方 | 0.4529 |  |  |  |  |  |
| 標準誤 | 34.40034 |  |  |  |  |  |
| 觀察值個數 | 135 |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |
|  | 自由度 | SS | MS | F | 顯著值 |  |
| 迴歸 | 10 | 143103.8 | 14310.38 | 12.09277 | 2.15E-14 |  |
| 殘差 | 124 | 146739.5 | 1183.383 |  |  |  |
| 總和 | 134 | 289843.3 |  |  |  |  |
|  | 係數 | 標準誤 | t 統計 | P-值 | 下限 95% | 上限 95% |
| 截距 | 225.002 | 11.40041 | 19.73631 | 4.47E-40 | 202.4374 | 247.5666 |
| x1 | -20.1325 | 3.10273 | -6.48864 | 1.87E-09 | -26.2737 | -13.9913 |
| x2 | -9.1314 | 3.10273 | -2.94302 | 0.003881 | -15.2726 | -2.99023 |
| x3 | 6.665651 | 3.10273 | 2.148318 | 0.033633 | 0.52448 | 12.80682 |
| x1x2 | 26.11111 | 4.054118 | 6.440639 | 2.37E-09 | 18.08688 | 34.13535 |
| x2x3 | 12.77778 | 4.054118 | 3.151802 | 0.002035 | 4.753542 | 20.80201 |
| x1x3 | -7.5 | 4.054118 | -1.84997 | 0.066698 | -15.5242 | 0.524236 |
| x1x2x3 | 3.055556 | 4.054118 | 0.753692 | 0.452463 | -4.96868 | 11.07979 |
| x1^2 | -12.363 | 4.660353 | -2.6528 | 0.009027 | -21.5871 | -3.13886 |
| x2^2 | -3.52634 | 4.660353 | -0.75667 | 0.450684 | -12.7505 | 5.697802 |
| x3^2 | -1.56264 | 4.660353 | -0.33531 | 0.737961 | -10.7868 | 7.661504 |

First, the table shows that E1,E2,E3,E12,E23,E12 reject the null hypothesis with α=0.05. Therefore, they are significant effects. Second, look closely on the ANOVA table, the value of F-test is larger than the critical value(“顯著值”). This indicates that the observed differences between tests are not caused by noise. Finally, the R2 and adjusted R2 are 0.493728 and 0.4529 respectively. This implies that the proportion of the variation explained by model is almost half.

Based on the first regression analysis table, a predictive regression model of the paper frogs’ jumping distance can be constructed:

y=b0+b1x1+b2x2+b3x3+b12x1x2+b23x2x3+b12x12

Final regression analysis:

| 摘要輸出 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 迴歸統計 |  |  |  |  |  |  |
| R 的倍數 | 0.689185 |  |  |  |  |  |
| R 平方 | 0.474977 |  |  |  |  |  |
| 調整的 R 平方 | 0.450366 |  |  |  |  |  |
| 標準誤 | 34.4799 |  |  |  |  |  |
| 觀察值個數 | 135 |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |
|  | 自由度 | SS | MS | F | 顯著值 |  |
| 迴歸 | 6 | 137668.8 | 22944.8 | 19.29978 | 6.18E-16 |  |
| 殘差 | 128 | 152174.5 | 1188.864 |  |  |  |
| 總和 | 134 | 289843.3 |  |  |  |  |
|  | 係數 | 標準誤 | t 統計 | P-值 | 下限 95% | 上限 95% |
| 截距 | 218.5916 | 4.274484 | 51.13872 | 4.69E-87 | 210.1338 | 227.0494 |
| x1 | -20.1325 | 3.109906 | -6.47366 | 1.86E-09 | -26.286 | -13.979 |
| x2 | -9.1314 | 3.109906 | -2.93623 | 0.00394 | -15.2849 | -2.97792 |
| x3 | 6.665651 | 3.109906 | 2.143361 | 0.033976 | 0.512171 | 12.81913 |
| x1x2 | 26.11111 | 4.063495 | 6.425776 | 2.36E-09 | 18.07079 | 34.15143 |
| x2x3 | 12.77778 | 4.063495 | 3.144529 | 0.002068 | 4.737458 | 20.8181 |
| x1^2 | -10.4119 | 3.378721 | -3.0816 | 0.002521 | -17.0972 | -3.72648 |

The analysis table shows that all effects in the predictive model are significant. The F-test value in ANOVA table is still larger than the critical value, implying that the observed differences between vary factors are statistically significant and not merely due to random noise. The R2 and adjusted R2 are 0.474977 and 0.450366, respectively.

Compare to the predictive model built in (6), several similarities and differences are observed.

Predictive model in (6):

y=b0+b1x1+b2x2+b12x1x2+b23x2x3+b13x1x3= 214.4444444-34.72222222x1-19.44444444x2+26.11111111x1x2+12.77777778x2x3-7.5x1x3

Predictive model in (15):

y=b0+b1x1+b2x2+b3x3+b12x1x2+b23x2x3+b12x12=218.5916294-20.13248291x1-9.131397298x2+6.665650756x3+26.11111111x1x2+12.77777778x2x3-10.41186057x12

- Similarity:

- Both models identify x1(length of papers), x2(width ratio of papers), x1x2(interaction effect between length and width ratio), andx2x3(interaction effect between width ratio and leg length) as significant effects. This suggests that these factors and their interactions effects are crucial in determining the jumping distance of the paper frogs.

- Difference:

- The current model includes x_3(length of the paper frog’s legs) as a significant factor. This highlights that the leg length of the paper frog plays a vital role in influencing the jumping distance, which was not directly considered in the previous model.

- The current model considers the quadratic term of x_1, indicating a nonlinear relationship between frogs’ jumping distance and papers length.

- Comparing to the previous model, the coefficients for x_1 and x_2 are significantly reduced in the current model. This suggests that: while x_1 and x_2 remain important, other factors or interactions might play a more dominant role in determining the jumping distance in the current context.

Suppose that residuals are normal distributed. The parameters of normal distribution are e and σe:

e=i=115j=19eij135=-1.26319E-15

σe=i15j9eij-e2135-1=33.69912197

- Rank each residual from 1~135

- The c.d.f. of i th effect is: pi=(i-0.5)/135

- The normal distribution value of i th effect: N-1(pi;e,σe)

Residual Q-Q plot:

Plot the residuals vs. run orders:

Plot the residuals vs. y:

Plot the residuals vs. y:

Plot the residuals vs. x1:

Plot the residuals vs. x2:

Plot the residuals vs. x3:

SN ratio: Larger-the-best

η=-10log101ni=1n1Xi2

|  |  |  |  | replicate 1 |  |  | replicate 2 |  |  | replicate 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | x1 | x2 | x3 | run1 | run2 | run3 | run1 | run2 | run3 | run1 | run2 | run3 | SN |
| 1 | -1 | -1 | -1 | 255 | 300 | 285 | 240 | 310 | 300 | 280 | 300 | 360 | 49.15775 |
| 2 | 1 | -1 | -1 | 200 | 205 | 230 | 220 | 155 | 200 | 150 | 165 | 200 | 45.37338 |
| 3 | -1 | 1 | -1 | 150 | 175 | 160 | 190 | 150 | 190 | 220 | 195 | 205 | 44.96722 |
| 4 | 1 | 1 | -1 | 170 | 210 | 160 | 160 | 170 | 185 | 150 | 155 | 200 | 44.62473 |
| 5 | -1 | -1 | 1 | 300 | 290 | 345 | 230 | 285 | 270 | 275 | 295 | 385 | 49.21638 |
| 6 | 1 | -1 | 1 | 140 | 150 | 145 | 200 | 150 | 185 | 145 | 140 | 135 | 43.57489 |
| 7 | -1 | 1 | 1 | 175 | 210 | 230 | 230 | 210 | 245 | 245 | 210 | 275 | 46.87 |
| 8 | 1 | 1 | 1 | 180 | 175 | 205 | 200 | 250 | 205 | 200 | 175 | 205 | 45.85851 |
| 9 | -1.682 | 0 | 0 | 210 | 220 | 215 | 145 | 160 | 150 | 165 | 175 | 175 | 44.79915 |
| 10 | 1.682 | 0 | 0 | 195 | 205 | 215 | 170 | 190 | 165 | 155 | 165 | 170 | 45.01357 |
| 11 | 0 | -1.682 | 0 | 205 | 185 | 180 | 175 | 185 | 225 | 225 | 180 | 205 | 45.74185 |
| 12 | 0 | 1.682 | 0 | 200 | 190 | 205 | 225 | 195 | 220 | 240 | 235 | 220 | 46.5465 |
| 13 | 0 | 0 | -1.682 | 205 | 190 | 220 | 210 | 230 | 200 | 175 | 155 | 170 | 45.60465 |
| 14 | 0 | 0 | 1.682 | 195 | 220 | 210 | 230 | 245 | 245 | 225 | 230 | 240 | 47.04125 |
| 15 | 0 | 0 | 0 | 250 | 230 | 260 | 210 | 260 | 225 | 200 | 200 | 220 | 47.0485 |

Check the correlation coefficient between each factor (including quadratic terms):

|  | x1 | x2 | x3 | x1x2 | x2x3 | x1x3 | x1x2x3 | x1^2 | x2^2 | x3^2 | SN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x1 | 1 |  |  |  |  |  |  |  |  |  |  |
| x2 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| x3 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |
| x1x2 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |
| x2x3 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |
| x1x3 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |
| x1x2x3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |
| x1^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |
| x2^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.3834 | 1 |  |  |
| x3^2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -0.3834 | -0.3834 | 1 |  |
| SN | -0.47446 | -0.16614 | 0.173636 | 0.480281 | 0.290149 | -0.15031 | 0.070694 | -0.28966 | 0.056844 | 0.106889 | 1 |

The result shows that there is little correlation between each factor, therefore, it can be considered as no multicollinearity effect among this regression models:

η=b0+b1x1+b2x2+b3x3+b12x1x2+b23x2x3+b13x1x3+b123x1x2x3+b12x12+b22x22+b32x32

Regression analysis results:

| 摘要輸出 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 迴歸統計 |  |  |  |  |  |  |  |  |
| R 的倍數 | 0.845818 |  |  |  |  |  |  |  |
| R 平方 | 0.715409 |  |  |  |  |  |  |  |
| 調整的 R 平方 | 0.00393 |  |  |  |  |  |  |  |
| 標準誤 | 1.584947 |  |  |  |  |  |  |  |
| 觀察值個數 | 15 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | 自由度 | SS | MS | F | 顯著值 |  |  |  |
| 迴歸 | 10 | 25.25935 | 2.525935 | 1.005524 | 0.545936 |  |  |  |
| 殘差 | 4 | 10.04823 | 2.512058 |  |  |  |  |  |
| 總和 | 14 | 35.30758 |  |  |  |  |  |  |
|  | 係數 | 標準誤 | t 統計 | P-值 | 下限 95% | 上限 95% | 下限 85.0% | 上限 85.0% |
| 截距 | 46.95483 | 1.575774 | 29.79795 | 7.55E-06 | 42.57978 | 51.32988 | 44.1528 | 49.75686 |
| x1 | -0.76285 | 0.428862 | -1.77878 | 0.149897 | -1.95356 | 0.427861 | -1.52545 | -0.00025 |
| x2 | -0.26713 | 0.428862 | -0.62288 | 0.567118 | -1.45784 | 0.923583 | -1.02973 | 0.495471 |
| x3 | 0.279175 | 0.428862 | 0.650968 | 0.55057 | -0.91154 | 1.469886 | -0.48342 | 1.041774 |
| x1x2 | 1.008985 | 0.560363 | 1.800591 | 0.146138 | -0.54683 | 2.564804 | 0.012551 | 2.005419 |
| x2x3 | 0.609552 | 0.560363 | 1.087779 | 0.337848 | -0.94627 | 2.16537 | -0.38688 | 1.605986 |
| x1x3 | -0.31577 | 0.560363 | -0.5635 | 0.603183 | -1.87158 | 1.240052 | -1.3122 | 0.680668 |
| x1x2x3 | 0.148515 | 0.560363 | 0.265033 | 0.804081 | -1.4073 | 1.704334 | -0.84792 | 1.144949 |
| x1^2 | -0.62719 | 0.644158 | -0.97366 | 0.385359 | -2.41566 | 1.161278 | -1.77263 | 0.518246 |
| x2^2 | -0.18966 | 0.644158 | -0.29444 | 0.783073 | -1.97813 | 1.598806 | -1.3351 | 0.955773 |
| x3^2 | -0.12647 | 0.644158 | -0.19634 | 0.853918 | -1.91494 | 1.661996 | -1.27191 | 1.018964 |

The result shows that all factors including quadratic term accept H0(null effect) with α=0.05, consequently, no regression model can be built. Therefore, the alpha value was rose to 0.15 in the hypothesis test to distinguish significant effect. As a result, E_{x_1} and E_{x_1x_2} are significant in this table. The F-test value is larger than critical value,